UART PROTOCOL

//BAUD_RATE

module baud_rate(input clk, rst, output reg tx_br, rx_br);

parameter clk_freq = 100;

parameter baud_rate=96;

parameter TX_MAX_COUNT= clk_freq/baud_rate;// 100mhz/9600

parameter RX_MAX_COUNT= clk_freq/baud_rate;

reg[31:0] tx_counter; // reg[$clog2(TX_MAX_COUNT) 1:0)

reg[31:0] rx_counter;

//TX BAUD-RATE

always@(posedge clk or posedge rst)begin

if(rst)begin

tx_br<=0;

tx_counter <=0;

end

else begin

if(tx_counter == (TX_MAX_COUNT)) begin

tx_br <=1;

tx_counter <=32'd0;

end

else begin

tx_counter <= tx_counter+1;

tx_br <=0;

end

end

end

always@(posedge clk or posedge rst)begin

if(rst)begin

rx_br<=0;

rx_counter<=32'd0;

end

else begin

if(rx_counter ==(RX_MAX_COUNT))begin

rx_br <=1;

rx_counter <=32'd0;

end

else begin

rx_br <=0;

rx_counter <= rx_counter+1;

end

end

end

endmodule

//BAYD RATE TB:

module baud_tb( );

reg clk,rst;

wire tx_br;

wire rx_br;

baud_rate dut(clk,rst,tx_br,rx_br);

always #5 clk=~clk;

initial begin

clk=0;rst=1;

#10 rst=0;

#900 $finish;

end

endmodule

---------------------------------------------------------

//UART_TX

module uart_tx(input clk,

input en,

input start,

input tx_br,

input [7:0]data,

output reg done,

output reg busy,

output reg tx_out  );

parameter RESET =0;

parameter IDLE=1;

parameter START=2;

parameter DATA=3;

parameter PARITY=4;

parameter STOP =5;  //1012

// INTERNAL REG

reg [7:0]hs_reg;  //hold&shift reg

reg [2:0] state;

reg parity ;

reg [2:0]bit_index;  //to count the data bits

always @ (posedge clk)begin

if(!en)begin

state<=RESET;

end

else if(tx_br)begin

case(state)

RESET:begin

busy<=0;

done<=0;

bit_index<=0;

tx_out<=1;  //always the transmission line of UART will be high

state<=IDLE;

end

IDLE:begin

if(start)begin

hs_reg <=data;  //load the data to hold register

parity <=^data;

state <=START;

end

else

state<=IDLE;

end

START:begin

tx_out <=0;

busy <=1;

done=0;

state<=DATA;

end

DATA:begin

tx_out<=hs_reg[0];

hs_reg<={1'b0,hs_reg[7:1]}; //hs reg>>1;

bit_index <=bit_index+1;

if(bit_index==7)

state<=PARITY;

else

state<=DATA;

end

PARITY:begin

tx_out<=parity;

state<=STOP;

end

STOP:begin

tx_out <=1;

busy <=0;

done <=1;

if(start)

state<=IDLE;

else

state<=RESET;

end

endcase

end

end

endmodule

//UART TX TB:

module uart_tx_tb( );

reg clk,en,start;

reg[7:0]data;

wire done;

wire busy;

wire tx_out;

uart_tx dut(.clk(clk),

.en(en),

.start(start),

.data(data),

.done(done),

.busy(busy),

.tx_out(tx_out));

always #5 clk=~clk;

initial begin

//$monitor("clk=%0b;en=%b;start=%b;data=%b;done=%b;busy=%b;tx out=%b",clk,en,start,data,done,bust,tx_out);

clk=0;en=0;data=8'b00000000;start=0;

#10 en=1;

data=8'b11100011;

start=1;

wait(done);

#10 start=0;

#10 start=1;data=8'b11001100;

wait(done);

#10 start=0;

#200 $finish;

end

endmodule

------------------------------------------------------------------

//UART_RX

module uart_rx(input clk,

input en,

input rx_br,

input rx_data,  //1-bit

output reg[7:0] rx_out,  //8-bit

output reg done,

output reg busy,

output reg error );

parameter RESET=0;

parameter START=1;

parameter DATA=2;

parameter PARITY=3;

parameter STOP=4;

parameter READY=5;

reg [2:0]state;

reg [7:0] sb_reg;  //shift & buff reg

reg [2:0]bit_index;

always @(posedge clk)begin

if(!en)begin

state<=RESET;

end

else if(rx_br) begin

case(state)

RESET:begin

done <=0;

busy<=0;

error <=0;

sb_reg<=0;

//rx_data<=0;

bit_index<=0;

state <=START;

end

START:begin

if(!rx_data)begin  //indicate the start bit

busy <=1;

state<=DATA;

end

else

state<=START;

end

DATA:begin

sb_reg <={rx_data,sb_reg[7:1]};

bit_index<=bit_index+1;

if (bit_index==7)

state <=PARITY;

else

state<=DATA;

end

PARITY:begin

error <=(^sb_reg != rx_data);

state <= STOP;

end

STOP:begin

if(rx_data)begin

done<=1;

rx_out <=sb_reg;

state<=READY;

end

else begin

error<=1;

state<=RESET;

end

end

READY:begin

busy <=0;

state<=RESET;

end

endcase

end

end

endmodule

//UART RX TB:

module uart_rx_tb(  );

reg clk,en,rx_data;

wire [7:0]rx_out;

wire busy,done,error;

uart_rx dut(.clk(clk),

.en(en),

.rx_data(rx_data),

.rx_out(rx_out),

.busy(busy),

.done(done),

.error(error));

task send_data(input [7:0]data,input parity);

integer i;

begin

rx_data = 0;

#10;

for(i=0;i<8;i=i+1)begin

rx_data=data[i];

#10;

end

rx_data=parity;

#10;

rx_data=1;  //stopbit

#10;

end

endtask

always #5 clk=~clk;

initial begin

$monitor("clk:%0b;en:%b;rx-data:%b;sb-reg:%b;rx-out:%b;done:%b;busy:%b;error:%b",

clk,en,rx_data,dut.sb_reg,rx_out,done,busy,error);

clk=0;en=0;rx_data=1;

#10 en=1;

#10;

send_data(8'b010101101,0);

#10;

$display("test completed");

$display("received data:%b",rx_out);

$stop;

end

endmodule

--------------------------------------------------------------

//TOP MODULE

module uart_top(input clk,

input rst,

input en,

input start,

input [7:0]tx_data,

output [7:0] rx_data,

output tx_done,tx_busy,

output rx_done,rx_busy,

output rx_error );

wire tx_out; //connecting output from tx to input of rx

wire tx_br,rx_br;

baud_rate br(.clk(clk),.rst(rst),.tx_br(tx_br),.rx_br(rx_br));

uart_tx tx(.clk(clk),.en(en),.start(start),.tx_br(tx_br),.data(tx_data),.tx_out(tx_out),.done(tx_done),.busy(tx_busy));

uart_rx rx(.clk(clk),.en(en),.rx_br(rx_br),.rx_data(tx_out),.rx_out(rx_data),.done(rx_done),.busy(rx_busy),.error(rx_error));

endmodule

//TOP MODULE TB:

module top_tb();

reg clk;

reg rst;

reg en;

reg start;

reg [7:0]tx_data;

wire [7:0] rx_data;

wire tx_done,tx_busy;

wire rx_done,rx_busy;

wire rx_error ;

uart_top dut(.clk(clk),

.rst(rst),

.en(en),

.start(start),

.tx_data(tx_data),

.rx_data(rx_data),

.tx_done(tx_done),

.rx_done(rx_done),

.tx_busy(tx_busy),

.rx_busy(rx_busy),

.rx_error(rx_error));

always #5 clk=~clk;

initial begin

$monitor("rx_data:%b, rx_data:%b, tx_done:%b, rx_done:%b, tx_busy:%b, rx_busy:%b",

tx_data, rx_data, tx_done, rx_done,tx_busy, rx_busy);

clk=0;rst=1;en=0;start=0;tx_data=8'd0;

#20 rst=0;en=1;

#20 t_bytes(8'b10110011);

wait(rx_done);

#20 ;

t_bytes (8'b11001010);

wait(tx_done);

#40;

$finish;

end

task t_bytes(input [7:0]data);

begin

wait(!tx_busy);

tx_data = data;

start=1;

#30 start=0;

end

endtask

endmodule